CS281

Virtual Development Board

Technical Reference Manual

RISC-V RV32IM Educational Platform

Version 1.0  —  Drexel University  —  CS281 Computer Architecture

# Table of Contents

# 1.  Introduction

The CS281 Virtual Development Board is a RISC-V RV32IM microcontroller platform emulated in Renode. It provides a realistic, bare-metal programming environment without requiring physical hardware. The board is purpose-built for the CS281 Computer Architecture course at Drexel University.

Students write programs in RISC-V assembly (and optionally C) that run directly on the emulated processor. Every peripheral is accessed through memory-mapped registers at fixed addresses, exactly as on a physical chip. The Renode simulator executes unmodified ELF binaries and exposes GDB-based debugging so students can pause execution, inspect registers, and step through code.

Figure 1 — CS281 Virtual Development Board (PCB rendering)

## 1.1  Processor Overview

| Parameter | Value |
| --- | --- |
| Architecture | RISC-V (RV32IM + Zicsr) |
| Data width | 32-bit |
| Extensions | M — integer multiply/divide; Zicsr — CSR instructions |
| Privilege level | Machine mode (M-mode) only |
| Byte order | Little-endian |
| Reset address | Set by ELF entry point (_start at 0x20000000) |
| Interrupt model | CLINT (software + timer) + external IRQs via cpu @ n |

NOTE  RV32I instructions always take priority in decoding. The M extension adds mul, mulh, div, divu, rem, remu. The Zicsr extension adds csrr, csrw, csrrs, etc. Compressed (C) and Atomic (A) extensions are NOT present; do not use them.

# 2.  Memory Map

All addresses are physical addresses on the system bus. The processor accesses every peripheral through normal load/store instructions (lw, sw, lb, sb, etc.) to the addresses listed below.

| Base Address | Name | Size | Access | Description |
| --- | --- | --- | --- | --- |
| 0x20000000 | ROM (Flash) | 256 KB | R / X | Program code and read-only data. Loaded from ELF at startup. Treated as read-only by the linker. |
| 0x40000000 | RAM | 512 KB | R / W / X | Initialized data (.data), zero-initialized data (.bss), heap, and stack. |
| 0xe0001800 | UART | 32 bytes | R / W | LiteX serial UART — main console I/O. |
| 0xe0002000 | Timer | 32 bytes | R / W | LiteX countdown timer with interrupt output. |
| 0xe0004000 | Buzzer GPIO | 4 bytes | W | Single-bit GPIO output driving the board buzzer. |
| 0xe0004400 | 7-Segment GPIO | 4 bytes | W | 8-bit GPIO output driving the 7-segment display (segments a–g + dp). |
| 0xe0005000 | CLINT | 64 KB | R / W | Core-Level Interruptor: machine software (msip) and machine timer (mtime / mtimecmp). |
| 0xe0015000 | GPIO Output | 4 bytes | W | Four LED outputs (bits 0–3 = LED0–LED3). |
| 0xe0015400 | GPIO Input | 4 bytes | R | Four board inputs: BTN0, BTN1, DIP_SW0, DIP_SW1 (bits 0–3). |

## 2.1  Memory Sections

The linker script (hardware/lib/cs281.ld) maps program sections to the memory regions above. Understanding this layout is essential for writing correct linker scripts and startup code.

| ELF Section | Region | Description |
| --- | --- | --- |
| .text.startup | ROM (first) | startup.S — must be at the very start of ROM so _start lands at 0x20000000. |
| .text | ROM | All other compiled/assembled code. |
| .rodata | ROM | String literals and other read-only constants. |
| .data | ROM → RAM | Initialized global/static variables. Stored (LMA) in ROM after .rodata; copied to RAM (VMA) by startup.S at boot. |
| .bss | RAM | Zero-initialized globals and statics. Not stored in ROM; startup.S writes zeros to this region. |
| (stack) | RAM (top) | Stack grows downward from _stack_top = 0x40080000 (top of RAM). No explicit section — the linker exports the symbol only. |

# 3.  Boot Sequence and Programmer's Model

When a program is loaded into Renode (via sysbus LoadELF), the simulator sets the CPU program counter to the ELF entry point. The linker script ensures this is _start, the first instruction of startup.S, located at 0x20000000.

## 3.1  Startup Sequence (startup.S)

The startup file in hardware/lib/startup.S performs four steps before calling main():

| Step | Action | Details |
| --- | --- | --- |
| 1 | Set stack pointer | la sp, _stack_top loads 0x40080000 into sp. The stack grows downward into RAM. |
| 2 | Copy .data to RAM | Reads from ROM (LMA = _etext) and writes to RAM (VMA = _sdata … _edata). Required because .data has initial values that must be in RAM at runtime. |
| 3 | Zero .bss | Writes zero words from _sbss to _ebss. The C standard requires zero-initialised globals; the ELF does not store zeros, so startup must do it. |
| 4 | Call main() | call main. If main() returns (it should not), startup spins in an infinite loop (j .Lhalt). |

## 3.2  Linker Symbols

These symbols are defined by cs281.ld and used by startup.S. They are available in any assembly or C file.

| Symbol | Value | Meaning |
| --- | --- | --- |
| _etext | (end of .rodata in ROM) | LMA (load address) of the .data section — where .data is stored in ROM. |
| _sdata | 0x40000000 + offset | VMA (run address) start of .data in RAM. |
| _edata | 0x40000000 + offset | VMA end of .data in RAM. |
| _sbss | (after .data in RAM) | Start of .bss region. |
| _ebss | (after .bss in RAM) | End of .bss region. |
| _stack_top | 0x40080000 | Top of RAM — initial value of the stack pointer. |

NOTE  _etext marks the boundary between code+rodata and the stored .data image in ROM. It is NOT simply the end of .text — it is the end of all ROM content.

# 4.  Interrupt Reference

The CS281 board uses RISC-V machine-mode (M-mode) interrupts. The CPU's mstatus.MIE bit globally enables interrupts; mie controls individual sources. When an interrupt fires, the CPU jumps to the address in mtvec and the mcause register identifies the source.

| CPU Line | mcause | Source | Type | Clear Procedure |
| --- | --- | --- | --- | --- |
| 3 | 0x80000003 | CLINT MSIP | Machine Software | Write 0 to CLINT_BASE + 0x0000 (MSIP register). |
| 7 | 0x80000007 | CLINT MTIP | Machine Timer | Write a new value to MTIMECMP greater than the current MTIME. |
| 11 | 0x8000000B | LiteX Timer | Machine External | Write 1 to Timer EV_PENDING (offset 0x3C) to acknowledge. |
| 12 | 0x8000000C | GPIO Input | Machine External | Read GPIO_IN to acknowledge. Change must be observed; no separate pending register. |

## 4.1  Enabling Interrupts (Assembly)

Minimal sequence to enable machine-mode interrupts in assembly:

# Write the trap vector address to mtvec

la   t0, trap_handler

csrw mtvec, t0

# Enable individual interrupt sources in mie

# Bit  3 = MSIE (software), bit 7 = MTIE (timer), bit 11 = MEIE (external)

li   t0, (1 << 11) | (1 << 7) | (1 << 3)

csrw mie, t0

# Set mstatus.MIE (bit 3) to globally enable interrupts

li   t0, (1 << 3)

csrs mstatus, t0

## 4.2  Trap Handler Template

trap_handler:

# Save caller-saved registers to the stack

addi sp, sp, -64

sw   ra,  0(sp)

sw   t0,  4(sp)

# ... save t1-t6, a0-a7 as needed ...

# Read mcause to identify the interrupt

csrr t0, mcause

# bit 31 = 1 for interrupts, 0 for exceptions

# bits 30:0 = interrupt/exception code (see table above)

# ... dispatch to handler ...

# Restore and return

lw   ra,  0(sp)

lw   t0,  4(sp)

addi sp, sp, 64

mret            # return to interrupted instruction

# 5.  UART — Serial Console

The UART peripheral provides a bidirectional serial character interface. It is the primary means of text I/O. In Renode it is connected to a telnet socket on port 3456 (run make uart-connect in a second terminal to view output).

Base Address: 0xe0001800

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | RXTX | R / W | — | Write: transmit one byte (character). Read: receive one byte. |
| 0x04 | TXFULL | R | 0 | 1 = TX FIFO is full; do not write to RXTX until 0. |
| 0x08 | RXEMPTY | R | 1 | 1 = RX FIFO is empty; no data ready to read. |
| 0x0C | EV_STATUS | R | 0 | Raw interrupt status flags (TX ready, RX available). |
| 0x10 | EV_PENDING | R / W | 0 | Interrupt pending flags. Write 1 to a bit to clear it. |
| 0x14 | EV_ENABLE | R / W | 0 | Interrupt enable mask. Set bit to route event to the CPU. |

## 5.1  Transmit a Character (Polling)

li   t0, 0xe0001800      # UART base

.wait_tx:

lw   t1, 4(t0)           # read TXFULL

bnez t1, .wait_tx        # spin until space available

sw   a0, 0(t0)           # write character (in a0) to RXTX

## 5.2  Receive a Character (Polling)

li   t0, 0xe0001800      # UART base

.wait_rx:

lw   t1, 8(t0)           # read RXEMPTY

bnez t1, .wait_rx        # spin until byte arrives

lw   a0, 0(t0)           # read character into a0

NOTE  The library function uart_putchar(a0) and uart_puts(a0) in hardware/lib/uart.S implement polling transmit. Link uart.S into your project and call them directly.

# 6.  Timer

The LiteX_Timer is a 32-bit countdown timer running at 100 MHz. It counts down to zero and fires an interrupt on CPU line 11. Important: the peripheral uses an 8-bit CSR data width. The 32-bit LOAD and RELOAD values are each spread across four consecutive 8-bit sub-registers at 4-byte-aligned addresses, written most-significant byte first (sub-register 0 holds bits[31:24], sub-register 3 holds bits[7:0]). Single-bit control registers (EN, EV_PENDING, EV_ENABLE) each occupy one word.

Base Address: 0xe0002000    |    Frequency: 100,000,000 Hz    |    IRQ: CPU line 11

| Offset | Register | Width | Description |
| --- | --- | --- | --- |
| 0x00 | LOAD0 | 8-bit W | Countdown start value bits[31:24] (MSB) |
| 0x04 | LOAD1 | 8-bit W | Countdown start value bits[23:16] |
| 0x08 | LOAD2 | 8-bit W | Countdown start value bits[15:8] |
| 0x0C | LOAD3 | 8-bit W | Countdown start value bits[7:0] (LSB). Write all four LOAD sub-registers before enabling the timer. |
| 0x10 | RELOAD0 | 8-bit W | Auto-reload value bits[31:24] (MSB). If RELOAD is non-zero the timer reloads automatically; write 0 for one-shot mode. |
| 0x14 | RELOAD1 | 8-bit W | Auto-reload value bits[23:16] |
| 0x18 | RELOAD2 | 8-bit W | Auto-reload value bits[15:8] |
| 0x1C | RELOAD3 | 8-bit W | Auto-reload value bits[7:0] (LSB) |
| 0x20 | EN | 8-bit W | Timer enable. Write 1 to start counting, 0 to stop. |
| 0x3C | EV_PENDING | 8-bit RW | Interrupt pending flag. Write 1 in your ISR to clear (acknowledge) the interrupt. Failure to clear causes the CPU to re-enter the ISR immediately after mret. |
| 0x40 | EV_ENABLE | 8-bit W | Write 1 to route the timer interrupt to CPU line 11. Write 0 to mask it. |

## 6.1  Periodic Interrupt Timer — 1 second at 100 MHz

Write LOAD and RELOAD byte-by-byte, MSB first. 100,000,000 = 0x05F5E100: byte 0 = 0x05, byte 1 = 0xF5, byte 2 = 0xE1, byte 3 = 0x00. Also configure mtvec, mie bit 11, and mstatus bit 3 (see Section 4.1) before enabling the timer.

li   t0, 0xe0002000

# Enable interrupt routing

li   t1, 1

sw   t1, 0x40(t0)       # EV_ENABLE

# Write LOAD: 100,000,000 = 0x05F5E100  (MSB first)

li   t1, 0x05

sw   t1, 0x00(t0)       # LOAD0: bits[31:24]

li   t1, 0xF5

sw   t1, 0x04(t0)       # LOAD1: bits[23:16]

li   t1, 0xE1

sw   t1, 0x08(t0)       # LOAD2: bits[15:8]

sw   x0, 0x0C(t0)       # LOAD3: bits[7:0]  = 0

# Write RELOAD (same values — repeats every second)

li   t1, 0x05

sw   t1, 0x10(t0)       # RELOAD0

li   t1, 0xF5

sw   t1, 0x14(t0)       # RELOAD1

li   t1, 0xE1

sw   t1, 0x18(t0)       # RELOAD2

sw   x0, 0x1C(t0)       # RELOAD3 = 0

# Start the timer

li   t1, 1

sw   t1, 0x20(t0)       # EN

## 6.2  ISR Pattern — Servicing and Clearing the Timer Interrupt

Inside your trap handler check mcause for 0x8000000B (bit 31 set = interrupt, lower 11 = CPU line 11). After servicing, write 1 to EV_PENDING (offset 0x3C) to clear the flag. The write must happen before mret or the CPU immediately re-enters the ISR.

machine_trap_handler:

addi  sp, sp, -64

sw    ra,  0(sp)

sw    t0,  4(sp)        # ... save t1-t6, a0-a7 similarly ...

csrr  t0, mcause

li    t1, 0x8000000B   # bit31=interrupt, source=11 (LiteX Timer)

bne   t0, t1, .Ldone

# --- service the timer interrupt ---

# ... your code here ...

# Clear EV_PENDING  ← REQUIRED before mret

li    t0, 0xe000203C   # TIMER_BASE + 0x3C

li    t1, 1

sw    t1, 0(t0)

.Ldone:

lw    ra,  0(sp)

lw    t0,  4(sp)

addi  sp, sp, 64

mret

NOTE  Using cs281.inc: the equates TIMER_BASE, TIMER_LOAD0–LOAD3, TIMER_RELOAD0–RELOAD3, TIMER_EN, TIMER_EV_PENDING, and TIMER_EV_ENABLE are all defined in hardware/lib/cs281.inc. Pre-computed byte slices of TIMER_FREQ (TIMER_FREQ_B0–B3) are also available.

# 8.  GPIO Output — LEDs

The GPIO Output peripheral drives four on-board LEDs. A single 32-bit register controls all four outputs simultaneously using a bitmask. Bits 31:4 are ignored.

Base Address: 0xe0015000

## 8.1  DATA Register (offset 0x00)

| Bits | Name | Description |
| --- | --- | --- |
| 31:4 | — | Reserved. Always write 0. |
| 3 | LED3 | 1 = LED3 ON, 0 = LED3 OFF |
| 2 | LED2 | 1 = LED2 ON, 0 = LED2 OFF |
| 1 | LED1 | 1 = LED1 ON, 0 = LED1 OFF |
| 0 | LED0 | 1 = LED0 ON, 0 = LED0 OFF |

## 8.2  Usage Examples

li   t0, 0xe0015000

li   t1, 0x1          # turn ON LED0 only

sw   t1, 0(t0)

li   t1, 0x5          # turn ON LED0 + LED2  (bits 0 and 2)

sw   t1, 0(t0)

sw   zero, 0(t0)      # turn OFF all LEDs

NOTE  Writing a new value replaces the entire register. To set a single LED without disturbing others, read-modify-write: lw t2,0(t0) / or t2,t2,t1 / sw t2,0(t0).

# 9.  GPIO Input — Buttons and DIP Switches

The GPIO Input peripheral reads four digital inputs: two push buttons and two DIP switch positions. Any state change on any input triggers an interrupt on CPU line 12.

Base Address: 0xe0015400    |    IRQ: CPU line 12 (state change)

## 9.1  DATA Register (offset 0x00)  — Read Only

| Bits | Name | Active | Description |
| --- | --- | --- | --- |
| 31:4 | — | — | Reserved. Reads as 0. |
| 3 | DIP_SW1 | 1 | DIP switch 1 position. 1 = switch ON, 0 = switch OFF. |
| 2 | DIP_SW0 | 1 | DIP switch 0 position. 1 = switch ON, 0 = switch OFF. |
| 1 | BTN1 | 1 | Push button 1. 1 = pressed, 0 = released. |
| 0 | BTN0 | 1 | Push button 0. 1 = pressed, 0 = released. |

## 9.2  Polling Example — Wait for BTN0

li   t0, 0xe0015400

.wait_btn:

lw   t1, 0(t0)         # read GPIO_IN

andi t1, t1, 0x1       # mask BTN0 (bit 0)

beqz t1, .wait_btn     # loop until pressed

## 9.3  Driving Inputs from the Renode Monitor

GPIO inputs have no physical hardware; they are driven from the Renode monitor console. Connect to the monitor terminal (the terminal where Renode runs) and type:

gpio_in SetGPIO 0 true     # press BTN0

gpio_in SetGPIO 0 false    # release BTN0

gpio_in SetGPIO 2 true     # turn DIP_SW0 ON

NOTE  Pin numbers: 0 = BTN0, 1 = BTN1, 2 = DIP_SW0, 3 = DIP_SW1.

# 10.  7-Segment Display

The 7-segment display is driven by an 8-bit GPIO output register. Each bit independently controls one segment of the display. Digits are formed by writing the correct bitmask for the desired segments.

Base Address: 0xe0004400

## 10.1  Segment Bitmask (DATA register, offset 0x00)

| Bit | Name | Physical Segment | Description |
| --- | --- | --- | --- |
| 7 | SEG_DP | Decimal point | 1 = dot lit |
| 6 | SEG_G | Middle bar | 1 = lit |
| 5 | SEG_F | Top-left bar | 1 = lit |
| 4 | SEG_E | Bottom-left bar | 1 = lit |
| 3 | SEG_D | Bottom bar | 1 = lit |
| 2 | SEG_C | Bottom-right bar | 1 = lit |
| 1 | SEG_B | Top-right bar | 1 = lit |
| 0 | SEG_A | Top bar | 1 = lit |

## 10.2  Digit Patterns

| Digit | Hex Value | Segments Active |
| --- | --- | --- |
| 0 | 0x3F | A B C D E F |
| 1 | 0x06 | B C |
| 2 | 0x5B | A B D E G |
| 3 | 0x4F | A B C D G |
| 4 | 0x66 | B C F G |
| 5 | 0x6D | A C D F G |
| 6 | 0x7D | A C D E F G |
| 7 | 0x07 | A B C |
| 8 | 0x7F | A B C D E F G |
| 9 | 0x6F | A B C D F G |

## 10.3  Example — Display the digit "3"

li   t0, 0xe0004400

li   t1, 0x4F          # digit 3: A+B+C+D+G

sw   t1, 0(t0)

NOTE  The constants SEG7_DIGIT_0 through SEG7_DIGIT_9 are defined in hardware/lib/cs281.inc for use in assembly programs.

# 11.  Buzzer

The buzzer is a single-bit GPIO output. Writing 1 activates the buzzer; writing 0 deactivates it. To produce a tone, toggle the output rapidly in a loop or timer ISR.

Base Address: 0xe0004000

| Bits | Name | Description |
| --- | --- | --- |
| 31:1 | — | Reserved. Always write 0. |
| 0 | BUZZER | 1 = buzzer ON, 0 = buzzer OFF |

## 11.1  Example — 500 Hz Tone (busy-loop, approximate)

li   t0, 0xe0004000

li   t2, 1000          # toggle count

.tone_loop:

li   t1, 1

sw   t1, 0(t0)         # buzzer ON

li   a0, 5000          # half-period delay

call delay

sw   zero, 0(t0)       # buzzer OFF

li   a0, 5000

call delay

addi t2, t2, -1

bnez t2, .tone_loop

# 12.  Register Quick Reference

All addresses shown are absolute byte addresses. All registers are 32-bit; use lw / sw unless noted.

| Address | Name | R/W | Description |
| --- | --- | --- | --- |
| 0xe0001800 | UART_RXTX | R/W | UART data — write byte to TX, read byte from RX |
| 0xe0001804 | UART_TXFULL | R | 1 = TX FIFO full, do not write |
| 0xe0001808 | UART_RXEMPTY | R | 1 = RX FIFO empty, no data available |
| 0xe0001810 | UART_EV_PENDING | R/W | UART interrupt pending — write 1 to clear |
| 0xe0001814 | UART_EV_ENABLE | R/W | UART interrupt enable |
| 0xe0002000 | TIMER_LOAD | R/W | Load value MSB byte (LOAD0). Write LOAD0-3 MSB-first. See Section 6. |
| 0xe0002004 | TIMER_RELOAD | R/W | Reload value MSB byte (RELOAD0). Write RELOAD0-3 MSB-first. See Section 6. |
| 0xe0002020 | TIMER_EN | R/W | Timer enable (1=run) |
| 0xe0002024 | TIMER_UPDATE | W | Write 1 to latch current count into VALUE sub-registers. |
| 0xe0002028 | TIMER_VALUE | R | Current count (latch first) |
| 0xe000203C | TIMER_EV_PENDING | R/W | Timer interrupt pending — write 1 to clear |
| 0xe0002040 | TIMER_EV_ENABLE | R/W | Timer interrupt enable |
| 0xe0004000 | BUZZER | W | Bit 0: 1=ON, 0=OFF |
| 0xe0004400 | SEG7 | W | Bits 0-7: segments A-G, DP |
| 0xe0005000 | CLINT_MSIP | R/W | Software interrupt pending (bit 0) |
| 0xe0009000 | CLINT_MTIMECMP | R/W | Timer compare low word |
| 0xe000BFF8 | CLINT_MTIME | R | Current time low word |
| 0xe0015000 | GPIO_OUT | R/W | LED outputs — bits 0-3 = LED0-LED3 |
| 0xe0015400 | GPIO_IN | R | Board inputs — bits 0-3 = BTN0,BTN1,DIP_SW0,DIP_SW1 |